MORNING GLORY

YACHT SPECIFICATION

TYPE	Twin Screw Bermudian Ketch

FLAG	Bermuda

DIMENSIONS	Length O.A.	48.2m (158ft)

Length W.L.	37.8m (124ft)

Beam		9.2m (30.2ft)

Draft (keel up)	3.3m (10.9ft)

Draft (keel down)	7.7m (25.1ft)

TONNAGE	Gross	309

Displacement (light ship)	380 tonnes

YEAR BUILT	1993 (extensively refitted in 2010)

BUILDER	Perini Navi Spa, Italy

NAVAL ARCHITECT	Perini Navi Spa, Italy

EXTERIOR DESIGNER	Perini Navi Spa, Italy

INTERIOR DESIGNER	Perini Navi Spa, Italy

CONSTRUCTION	AH-36 Steel hull & 5083 aluminium alloy superstructure

Teak laid decks

CLASSIFICATION	ABS A1 Yachting Service +AMS +ABCU

MACHINERY

MAIN ENGINES	2 x 850hp MAN diesel engines type D2842 LE406

(new 2010)

GEAR BOXES	2 x ZF BW 191-1 4.0:1 Vertical offset type

(complete rebuild 2010)

SHAFTS & PROPELLERS	2 x BERG 360 DF/3 High skewed 1350mm 3 blades - with fixed pitch sail/motor options

SPEEDS & FUEL CONSUMPTIONS	Approximately 14.5 knots continuous with a fuel consumption of approx 120 litres/hour

(2 x main engines and 1 x 85 Kw generator)

RANGE	Approximately 4,900 NM

SAILING SYSTEMS

RIG	Ketch rigged in-mast furling

Complete new generation Perini Navi sail system with latest generation dashboards

RIG DIMENSIONS	Main mast - deck stepped 42.00m x 675mm with

4 x sets spreaders 1 x jumper strut & 12.70m boom

Mizzen mast – deck stepped 32.40 x 520mm with

3 x sets spreaders 1 x jumper strut & 9.00m boom

SAILS	Main 240sqm

Mizzen 135sqm

Genoa 340sqm

Staysail 129sqm

Fisherman 158sqm

SAIL AREAS	Total sail area 1002sqm

STANDING RIGGING	OYS Nitronic 50 stainless steel with stainless turnbuckles & stainless shroud rollers

RUNNING RIGGING	Mega-Twin Dyneema sheets & halyards with Vectran & Spectra utility lines, toppers & pendants

WINCHES	Bamar/Perini Navi new stainless steel furlers 2010

Perini Navi captives overhauled 2010/12

New line tensioners/load cells installed 2010

AUXILIARY MACHINERY & ELECTRICAL SYSTEM

GENERATORS				2 x 85kW Northern Lights diesel generators (new 2010)

1 x 55kW Northern Lights generator (new 2010)

ELECTRICITY	380VAC 3 Phase 50Hz, 220VAC 50Hz, 110VAC 50Hz,

24VAC 50Hz

SHORE POWER			80 KW/A Connection with 380VAC 3 Phase 50Hz

BATTERIES	24VDC 2 cell service & emergency batteries with Mastervolt charger

10 x 24VDC Deep cycle AGM start batteries with Mastervolt charger

8 x 24 VDC GMDSS Radio batteries with Sailor charger

TANK CAPACITIES (approximate)

FUEL	41,200 litres (10,899 US gallons)

WATER	12,000 litres (3,174 US gallons)

WATERMAKERS	2 x HEM watermakers with 10,000 litres (2,645 US gallons) capacity

LUB OIL	600 litres (158 US gallons)

DIRTY LUB OIL	800 litres (211 US gallons)

SLUDGE	2,130 litres (563 US gallons)

BILGE	2,130 litres (563 US gallons)

GREY WATER	2,000 litres (529 US gallons)

BLACK WATER	2,000 litres (529 US gallons)

ANCILLARY EQUIPMENT

STEERING SYSTEM	2 x Marsilli hydraulic/electric transmitter and manual/emergency override

BOW THRUSTER	1 x 55KW Schottel bowthruster type STT-010LK 500mm 4 Blades

FUEL FILTERS	Racor primary/secondary with cartridge backup

FUEL SEPERATOR	2 x Centrifugal fuel filters type Alfa Laval MIB 303S-13 new 2010

OIL/WATER SEPARATOR	1 x DVZ oil water separator type OMD-2005 (new 2010)

FIRE/BILGE PUMP	1 x 5Kw 380VAC Fire/1 x 5Kw 380VAC Bilge each with mutual redundancy (new 2010)

24VDC Automatic submersible pumps in each compartment

BOILERS	Dual element calorifier with on-demand hot water loop throughout

SEWAGE TREATMENT	HAMAN Automated sanitation system (new 2010)

FRESHWATER TREATMENT	HAMAN Automated silver ion sterilizer

Dual Gianneschi MAK 300D Automated pressure system

WC's	Dual Evac/Jets vacuum with premium sets (new 2010)

AIRCONDITIONING	2 x Copeland ZR 19M3 scroll type compressors

Condaria/MarineAir individual control units throughout

VENTILATION	Variable speed MarineAire makeup air units with pre heating/de- humidifier function

Delta-T supply and extraction with ActionAir fire dampers throughout

MAIN REEFERS	2,980 litres combination refrigeration/freezer capacity (new 2012)

SECONDARY REFRIGERATION	3 x 100 litres crew mess, bar & aft galley

SAFETY & SECURITY EQUIPMENT

Extensive personal lifesaving equipment

Statement of voluntary compliance to MCA Large Yacht Code

COMMUNICATION EQUIPMENT GMDSS EQUIPPED & CERTIFIED

SATCOM	2 x KVH Fleet FB500 Broadband

2 x Iridium satellite communications terminals

Sailor INMARSAT C

CELLULAR SYSTEM	2 x Globesurfer III 4G GSM system

VHF RADIOTELEPHONES	3 x Sailor RT4822 VHF radiotelephones with DSC

10 x Icom IC M1V Handheld VHF radiotelephones

4 x Icom IC-GM1600 GMDSS Handheld VHF radiotelephones

SSB RADIOTELEPHONE	Sailor HC 4500 MF/HF Transceiver

WATCHKEEPING RECEIVER 	Sailor RT4822 VHF DSC

TELEPHONE EXCHANGE	Panasonic PABX

INTERCOM SYSTEM	Panasonic all page/selective page

NAVIGATION EQUIPMENT

MAGNETIC COMPASS	C PLATH Jupiter 180mm gimbal ceiling mount

GYRO COMPASS	Type C PLATH NAVIGAT II

AUTOMATIC PILOT	Type C PLATH NAVIGAT II with EL RP & NFU

LOG	2 x Furuno FI 50 (new 2012)

ECHO SOUNDER	2 x Furuno LS4200 (new 2012)

RADAR	2 x Furuno S Band (new magnetrons 2012)

#### CHART PLOTTER	2 x Transas world folio with glass bridge arrangement

GPS	2 x Northstar

WEATHER FAX	Sailor DT4646E

NAVTEX	Furuno NX500

WIND INSTRUMENTS	Furuno FI 50 throughout

OTHER	Sailor BP 4365 Alarm panel

Sailor BP 4680 Battery panel

VEI Repeater screens

Axis video servers

DECK EQUIPMENT

ANCHOR WINDLASS	2 x Variable speed Perini Navi with wildcats

AFT WARPING CAPSTANS	2 x dual speed Perini Navi

GROUND TACKLE	2 x Perini Navi anchors with 5 shots stud link (90ft) ea

BOAT CRANE & DAVITS	Remote tender lifting pole (new 2012)

PASSERELLE	Motomar/Perini Navi teak/alloy/stainless extendable with elevation

SIDE BOARDING LADDERS	Motomar/Perini Navi starboard side to waterline

SWIMMING PLATFORM	Swimming ladder to side boarding & shell door platform on starboard side aft (with access from lazarette)

SEARCHLIGHT	ACR Main mast starboard spreader #1 with FB control

HORN	Dual Kahlenberg

AWNING	Flybridge & Cockpit removable (new 2012)

AUDIO/VISUAL ENTERTAINMENT SYSTEMS

Audio visual entertainment systems throughout the yacht new 2012

OFFICE EQUIPMENT

COMPUTERS		5 x Rack servers with wireless network & access controller

PHOTOCOPIER/PRINTER	Brother Wireless network printer/scanner

FACSIMILE	Brother

DOMESTIC EQUIPMENT

Miele induction 5 position hotplate (new 2010)

2 x Miele pyro lytic ovens (new 2010)

2 x Miele dishwashers (new 2010)

2 x Miele washers (new 2010)

2 x Miele dryers (new 2010)

TENDERS & WATERSPORTS EQUIPMENT

TENDERS	2 x 6.4m (21ft) Nautica tenders with Yanmar 250HP Hamilton jet units (new 2012)

WATERSPORTS	Bauer SCUBA compressor

EQUIPMENT	14 sets SCUBA equipment (new 2012)

Various inflatable tow toys (new 2012)

Wakeboards, ski’s knee boards etc (new 2012)

ACCOMMODATION

OWNER	1 x Full beam owner’s cabin aft with shower room and bath room en suite

GUESTS	2 x double guest cabins with shower room en suite

1 x twin guest cabin with shower room en suite

CREW	4 x double crew cabins with en suite

REFIT/WORKS LIST

Valuable works 2015-2016

Class special survey - first part

Tanks inspection, cleaning and painting

Deck equipment service (windlass, and deck winches)

New electronic devices (Gyro, Autopilot, Rudder feedback)

Main engines and generators service

New chain and swivels

New fire fighting system

Alarm sensor for watertight doors

Shafts and rudder service

Valuable works 2016-2017

Class special survey – second part

Stability test

Hull and superstructure paint

Hull bottom sandlblasting

Masts unstepping/stepping, service and painting

Led lights on mast spreaders

Rigging service

New main and mizzen foils

Captive winches and furlers service

Fly bridge extension

Forward deck and fly bridge new teak

Repair hull/bulwark plates

Electrical system overhaul

Systems control

Monitoring system upgrade

Technical bilges paint

New centerboard ram

New sinks for crew quarter

New owner shower area and box

External furniture varnishing

These particulars have been prepared by us from information provided by the sellers and are intended as a general guide to the vessel. We cannot, however, be liable for any inaccuracy. It is up to the purchaser to confirm details of concern to him by survey, inspection and/or enquiry of the seller and to ensure that the purchase contract properly reflects his concern and specific details upon which he may rely. We always advise an independent survey.